# GOME Level 1b to 1c conversion

We will focus very briefly on how to obtain GOME Level 1c data from Level 1b data. In other words, starting from uncalibrated data (containing the raw detector signals of the science measurements plus calibration constants), we want to obtain a new set containing data calibrated according to some chosen criteria.

The tool (or ‘processor’) we need for such conversion is called GDP01, a console application available for several platforms, such as Windows, Linux and Solaris. It requires no installation (you just need to launch the executable file) and it accepts as input an instrument binary Level 1b data file, usually with ‘lv1’ extension.

Basically, this file is a collection of spectral elements called ‘pixels’; every pixel is composed of four spectral channels. In our example we will focus on the pixel 1 in order to obtain small sized output files.

By using the ‘p’ parameter followed by two non-decreasing indexes we are able to specify a particular range of pixels for the extraction, instead of the default value (all available pixels). The ‘c’ parameter is instead used to select the preferred calibration steps to be performed, through a sequence of characters created according to page 24 of the manual.

The following command extracts the first ground pixel without applying any calibration step:

gdp01_ex.lin -p 1 1 -c '' 199607010116_06255.lv1 out

The output file (named ‘out.el1’) is roughly an ASCII printing of the raw measurements (level 1b).

The following command extracts instead the first ground pixel applying all the supported calibration steps (Leakage, FPA, Fixed, Straylight, Normalize, BSDF, Polarization, Intensity, Unit Conversion):

gdp01_ex.lin -p 1 1 -c 199607010116_06255.lv1 out-ca

The output is an ASCII printing of the fully calibrated data (level 1c).

Moreover, for instance,

gdp01_ex.lin -p 1 1 -c L 199607010116_06255.lv1 outl

extracts the first ground pixel applying only a Leakage calibration step on the input data.

A generic output file dump follows. As we can see, there is a header showing general information and, for each pixel and channel, a list of rows containing the values for wavelength, irradiance and so on.

/*---------------------------------------------------------------------------*\

**  GDP Level 0-to-1 Extracting * Version 2.21  * Copyright <A9> DLR 1996-2002

**

\*---------------------------------------------------------------------------*/

Calibrations Applied

Cross-Correlation

Units

Wavelength [-], Irradiance [-], Radiance [-]

E2GOM062550001GSLVL10 DP20061228095625

ERS Information

16983  3720000  06254   5651.170410  -4405.594727     -7.654252

(…)

Channel 1

237.0786 314.7213 0695 0.0177 0 0 0 0

237.0786 4.29422e+01 5.89848e-02 8.55734e-01 0

237.2012 4.92572e+01 6.11567e-02 9.81150e-01 0

237.3237 5.39127e+01 6.27308e-02 1.07363e+00 0

237.4462 3.82170e+01 5.68902e-02 7.61888e-01 0

237.5686 3.23108e+01 5.43258e-02 6.44637e-01 0

237.6909 3.98934e+01 5.64405e-02 7.95094e-01 0

237.8133 5.22547e+01 6.02066e-02 1.04058e+00 0

(…)

Channel 2

311.1835 405.1806 0841 0.0120 0 0 0 0

311.1835 0.00000e+00 0.00000e+00 -1.00000e+00 0

311.2999 0.00000e+00 0.00000e+00 -1.00000e+00 0

311.4164 0.00000e+00 0.00000e+00 -1.00000e+00 0

311.5327 0.00000e+00 0.00000e+00 -1.00000e+00 0

311.6491 0.00000e+00 0.00000e+00 -1.00000e+00 0

311.7654 0.00000e+00 0.00000e+00 -1.00000e+00 0

(…)

From the outputs, it is easy to extract sequences of pairs (e.g. [wavelength,irradiance]) that can be easily used to generate a bi-dimensional graph, e.g. using Microsoft Excel. Iterating such process for several data sets calibrated through different steps, and of course selecting every time the same wavelength range, we are thus able to show graphically the way a particular calibration steps sequence alters the original data set.

In conclusion, the use of GDP01 processor allows us to switch from binary to ASCII data and to change the measurements according to the selected calibration steps.